SANTA CLARA COUNTY BUILDING INSPECTION OFFICE

TYPICAL RESIDENTIAL INTERCONNECTED STORAGE BATTERY SYSTEM REQUIRED SIGNAGE

SIGNS SHALL BE WEATHERPROOF AND SUITABLE FOR THE ENVIRONMENT THEY ARE INSTALLED

LETTERING SHALL BE A MINIMUM LETTER HEIGHT OF 3/8” PERMANENTLY AFFIXED.

CAUTION STORAGE BATTERY SYSTEM

CONNECTED

Label Main Service: CEC 705.12(D)(4)

WARNING

INVERTER OUTPUT CONNECTION

DO NOT RELOCATE THIS

OVERCURRENT DEVICE

Label on POI breaker if OCPDs exceed 100% of bus rating CEC 705.12 (D) (7)

A PERMANENT PLAQUE OR DIRECTORY, DENOTING ALL ELECTRIC POWER SOURCES ON OR IN THE PROMISES SHALL BE INSTALLED AT EACH SERVICE EQUIPMENT LOCATION AND AT LOCATIONS OF ALL POWER SOURCES CAPABLE OF BEING INTERCONNECTED

To be applied at exterior of building in readily visible location.

A directory needs to be installed when utility service disconnect and

PV system disconnect are not in the same location.

Must be at the location of the photovoltaic system disconnecting means.

NEC 705.10

Current Codes and abbreviations

2013 California Electrical Code (CEC)

2013 California Residential Code (CRC)

2012 International Fire Code (IFC)

POI= Point of interconnect, OCPD= Over-Current Protection Device